# Floating Point and Fixed Point Interoperable ANSI C Implementations of BroadVoice16

/*****************************************************************************/

/* BroadVoice(R)16, Copyright (c) 2000-12, Broadcom Corporation.             */

/* All Rights Reserved.                                                      */

/*                                                                           */

/* This software is provided under the GNU Lesser General Public License,    */

/* version 2.1, as published by the Free Software Foundation ("LGPL").       */

/* This program is distributed in the hope that it will be useful, but       */

/* WITHOUT ANY SUPPORT OR WARRANTY; without even the implied warranty of     */

/* MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the LGPL for     */

/* more details.  A copy of the LGPL is available at                         */

/* http://www.broadcom.com/licenses/LGPLv2.1.php,                            */

/* or by writing to the Free Software Foundation, Inc.,                      */

/* 59 Temple Place - Suite 330, Boston, MA  02111-1307, USA.                 */

/*****************************************************************************/

# Scope

This document provides a description of two alternative interoperable implementations of the BroadVoice16 speech coding algorithm.  If you encounter any problems please send an email to broadvoice@broadcom.com

# References

[1]	American National Standard, ANSI/SCTE 24-21 2006, “BV16 Speech Coding Specification for Voice over IP and Cable Telephony Applications.”

[2]	RTP Payload Format for BroadVoice Speech Codecs, http://www.rfc-editor.org/rfc/rfc4298.txt

# BroadVoice16 Algorithm Description

The BroadVoice16 speech coding algorithm is described in ANSI standard: American National Standard, ANSI/SCTE 24-21 2006 [1], and the payload format is specified in RFC 4298 [2].

# BroadVoice16 Simulation Software

The BroadVoice16 software is available in both fixed point and floating point.  The two versions are available as ANSI C source code, generally with a one to one correspondence between the two versions.  The two versions provide interoperable implementations of BroadVoice16 as described in the ANSI 24-21 2006 standard.  Hence, unless otherwise noted, the following applies to both the fixed point and floating point software.  The fixed point and floating point software are located in the FixedPoint and FloatingPoint directories, respectively.  Prior to compiling the fixed point code it is necessary to download fixed point operator library files from ITU-T, see Section 5.1 for details.

A Microsoft Visual C++ 8.0 Express Edition project is located in the MSVC sub-directory.  To open the project and compile the code, open the MSVC BroadVoice16 solution file (this applies to both FixedPoint and FloatingPoint).

Makefiles for various platforms are present in the MKS, Linux, and Solaris sub-directories.  The MKS is for the Unix-like environment provided on Windows PCs with the MKS Toolkit.  It uses the Microsoft C compiler.  The second is named Linux and was developed for a 64-bit machine (little endian) running Linux.  The third is named Solaris and was developed for Solaris 32-bit x86 (little endian) machine.  It was also tested on a Solaris 64-bit Sparc (big endian) machine.  The makefiles in the Linux and Solaris sub-directories use the GNU gcc compiler.  In any of the three directories, the code can be compiled by simply issuing the make command.  Note that the above refer to platforms and compilers for which compilation and operation has been verified.  However, it should be simple to generalize makefiles and typedef.h types for other platforms.

Floating point and fixed point versions both include a script for very basic verification of correct compilation and operation of the software.  The script and test vectors are located in the sub-directory named “process”.   To execute the simple test, compile the code and execute the TestBroadVoice16 script.  Note that the test script requires an argument specifying endian format of processing platform:

Big endian platform:		TestBroadVoice16 BigEndian

Little endian platform: 	TestBroadVoice16 LittleEndian

To clean up files: 		TestBroadVoice16 CleanAll

The reference files are all stored in little endian format, and the TestBroadVoice16 script byte swabs the reference files prior to codec processing for “TestBroadVoice16 BigEndian”.

For proper verification of an actual implementation of BroadVoice16 (on a DSP or other processor) a much larger set of test vectors is required.  Such test vectors can be generated by passing input files through the BroadVoice16 fixed point ANSI C code and collecting the output files.  It is important to also include tests of the decoder with packet loss and bit-errors in the bit-stream.

The source code is by default (G192BITSTREAM=1) compiled to produce a bit-stream compliant to the format defined in ITU-T G.192.  This is a format convenient for simulations where channel impairments can be inserted in the bit-stream with typical tools used by the ITU-T for speech codec evaluations.  It is only for offline simulation purposes.  To get a packed bit-stream for real-world communication purposes the code should be compiled with G192BITSTREAM=0.

## Compile with G192BITSTREAM=1

This is the default compilation setting.  It can be changed in the makefiles or the MSVC project.

Similar to ANSI C software of ITU-T speech coders, the BroadVoice16 encoder executable takes 16-bit linear PCM as input and generates a bit-stream compliant with ITU-T G.192, and the BroadVoice16 decoder takes a ITU-T G.192 compliant bit-stream and generates a 16-bit linear PCM output.  All files are in endian format native to the processing platform.  The calling conventions are:

Encode: BroadVoice16 enc input_speech g192_bitstream

Decode: BroadVoice16 dec g192_bitstream output_speech

Since the simulation software supports the G.192 bitstream format, all the tools typically used for channel impairment simulations for ITU-T speech coders such as G.729 can also be used for the BroadVoice16 simulation software.

The TestBroadVoice16 script is based on default compilation (G192BITSTREAM=1).

## Compile with G192BITSTREAM=0

In Version 1.1 and onwards the packed bit-stream (compile with G192BITSTREAM=0) is big endian regardless of native endian format of processing platform, and hence the packed bit-stream is compliant with RFC 4298.  On a little endian platform this means that the packed bit-stream of Version 1.1 will be byte-swabbed when compared with Version 1.0.  On a big-endian platform there should be no difference.  To emphasize, the packed bit-stream of Version 1.1 onwards is directly compliant with RFC 4298 regardless of endian format of processing platform.  The calling conventions are:

Encode: BroadVoice16 enc input_speech rfc4298_bitstream

Decode: BroadVoice16 dec rfc4298_bitstream output_speech

Encoder input and decoder output are 16-bit linear PCM in endian format native to the processing platform, while the bit-stream is packed according to RFC 4298, and hence is always in big endian format.

Note that although the TestBroadVoice16 script is setup for compilation with G192BITSTREAM=1, very basic operation of compilation with G192BITSTREAM=0 can be verified by encoding and decoding of reference input speech used by TestBroadVoice16, and then check output speech against the reference output speech used by TestBroadVoice16.  Furthermore, note that for this manual verification of compilation with G192BITSTREAM=0, byte swabbing of little endian reference encoder input and decoder output signals will be necessary for big endian platforms (this is done automatically within TestBroadVoice16 for big endian platforms).

# C source Code for BroadVoice16

The fixed point and floating ANSI C codes attempt to maintain a one to one correspondence to facilitate implementation and work with the code.  An overview of the fixed point ANSI C code is provide in Section 5.1, and an overview of the floating point  ANSI C code is provided in Section 5.2.

Make sure that proper type definitions are defined in typedef.h for the specific compiler being used:

- UWord8:	unsigned 8-bit integer

- Word16: 	signed 16-bit integer

- Word32:	signed 32-bit integer

- UWord32:	unsigned 32-bit integer

## BroadVoice16 fixed point ANSI C code

An overview of the files for the ANSI-C BroadVoice16 fixed point C code is provided in Table 1 and Table 2.  The fixed point implementation is based on the ITU-T fixed point operator library of the ITU-T STL (Software Tool Library) and the ITU-T fixed point operator library of speech coding standard G.729 Annex I.  The two components of the library need to be downloaded separately from ITU-T (free download).  For detailed instructions of download procedure and URLs see the readme files in the sub-directories FixedPoint/itug191lib and FixedPoint/itug729ilib.

Table 1 BroadVoice16 fixed point C source files.

| File name | Description |
| --- | --- |
| Bitpack.c | Bit packing functions |
| Bv.c | Main for both encoder and decoder |
| Coarptch.c | Coarse pitch function |
| Decoder.c | Decoder function |
| Encoder.c | Encoder function |
| Excdec.c | Excitation decoding function |
| Excquan.c | Excitation quantization function |
| Fineptch.c | Pitch refinement function |
| G192.c | G.192 bit-stream related functions |
| Gaindec.c | Gain decoding function |
| Gainquan.c | Gain quantization function |
| Levelest.c | Level estimation function |
| Lspdec.c | LSP decoding function |
| Lspquan.c | LSP quantization function |
| Plc.c | Packetloss concealment function |
| Postfilt.c | Postfilter function |
| Preproc.c | Input preprocessing function |
| Ptquan.c | Pitch tap quantization function |
| Tables.c | Various tables |
| A2lsp.c | Predictor to LSP coefficient conversion function |
| Allpole.c | All-pole filtering functions |
| Allzero.c | All-zero filtering functions |
| Autocor.c | Autocorrelation function |
| Basop32.c | Include of downloaded ITU-T fixed basic operator .c files |
| Cmtables.c | Common tables |
| Mathutil.c | Additional math functions (power, log, and sqrt) |
| Mathtables.c | Tables for fixed point math functions |
| Levdur.c | Function for Levinson-Durbin recursion |
| Lsp2a.c | LSP to predictor coefficient conversion function |
| Memutil.c | Dynamic memory utility functions |
| Ptdec.c | Pitch tap decoding function |
| Stblzlsp.c | LSP stabilization function |
| Utility.c | Data copy utility functions |
| Vqdecode.c | General VQ decoding function |

Table 2 BroadVoice16 fixed point H source files

| File name | Description |
| --- | --- |
| Bitpack.h | Prototypes for bit packing functions |
| Bv16.h | Prototypes for main encoder and decoder functions |
| Bv16cnst.h | Constant values |
| Bv16externs.h | Extern tables and function prototypes |
| Bv16strct.h | Structures |
| G192.h | Prototypes for G.192 bit-stream related functions |
| Postfilt.h | Prototype for postfilter function |
| Basop32.h | Include of downloaded ITU-T fixed basic operator .h files |
| Bvcommon.h | General constants, externs, and prototypes |
| Mathutil.h | Externs and prototypes for fixed point math functions |
| Memutil.h | Prototypes for dynamic memory utility functions |
| Typedef.h | Type definitions |
| Utility.h | Prototypes for data copy utility functions |

## BroadVoice16 floating point ANSI C-code

An overview of the files for the ANSI-C BroadVoice16 floating point C code is provided in Table 3 and Table 4.

Table 3 BroadVoice16 floating point C source files

| File name | Description |
| --- | --- |
| Bitpack.c | Bit packing functions |
| Bv.c | Main for both encoder and decoder |
| Coarptch.c | Coarse pitch function |
| Decoder.c | Decoder function |
| Encoder.c | Encoder function |
| Excdec.c | Excitation decoding function |
| Excquan.c | Excitation quantization function |
| Fineptch.c | Pitch refinement function |
| G192.c | G.192 bit-stream related functions |
| Gaindec.c | Gain decoding function |
| Gainquan.c | Gain quantization function |
| Levelest.c | Level estimation function |
| Lspdec.c | LSP decoding function |
| Lspquan.c | LSP quantization function |
| Plc.c | Packetloss concealment function |
| Postfilt.c | Postfilter function |
| Preproc.c | Input preprocessing function |
| Ptquan.c | Pitch tap quantization function |
| Tables.c | Various tables |
| A2lsp.c | Predictor to LSP coefficient conversion function |
| Allpole.c | All-pole filtering functions |
| Allzero.c | All-zero filtering functions |
| Autocor.c | Autocorrelation function |
| Cmtables.c | Common tables |
| Levdur.c | Function for Levinson-Durbin recursion |
| Lsp2a.c | LSP to predictor coefficient conversion function |
| Ptdec.c | Pitch tap decoding function |
| Stblchck.c | LSP check function |
| Stblzlsp.c | LSP stabilization function |
| Utility.c | Data copy utility functions |

Table 4 BroadVoice16 floating point H source files

| File name | Description |
| --- | --- |
| Bitpack.h | Prototypes for bit packing functions |
| Bv16.h | Prototypes for main encoder and decoder functions |
| Bv16cnst.h | Constant values |
| Bv16externs.h | Extern tables and function prototypes |
| Bv16strct.h | Structures |
| G192.h | Prototypes for G.192 bit-stream related functions |
| Postfilt.h | Prototype for postfilter function |
| Bvcommon.h | General constants, externs, and prototypes |
| Typedef.h | Type definitions |
| Utility.h | Prototypes for data copy utility functions |

# Version History

The current version is Version 1.1.

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 |  | Broadcom | Initial Version |
| 1.1 | Nov 13, 2009 | Broadcom | - Added makefile compilation environments for MKS, Linux, Solaris. - Eliminated use of long type to facilitate proper operation on 64-bit platforms. - Changed includes to eliminate general include of .h files by other .h files. - Tested on 32-bit, 64-bit, little endian, and big endian machines. - Updated TestBroadVoice16 to support big endian platform in addition to little endian platform. - Changed packed bit-stream (compile with G192BISTREAM=0) endian format to always be big endian (RFC 4298) regardless of endian format of processing platform. |

Floating point a2lsp.c:

/*-------------- Telecommunications & Signal Processing Lab -------------------

*                             McGill University

* Copyright (c) 1994, P. Kabal

* Portions of this code are provided by P. Kabal and contributors "as

* is" and any express or implied warranties, including, but not limited to,

* the implied warranties of merchantability and fitness for a particular

* purpose are disclaimed. In no event shall the copyright holder or

* contributors be liable for any direct, indirect, incidental, special,

* exemplary, or consequential damages (including, but not limited to,

* procurement of substitute goods or services; loss of use, data, or

* profits; or business interruption) however caused and on any theory of

* liability, whether in contract, strict liability, or tort (including

* negligence or otherwise) arising in any way out of the use of this

* software, even if advised of the possibility of such damage.